# Developer’s Guide:

# SMS with the M1

1. Introduction	3

2. Overview:	4

2.1 SMS-DELIVER (Mobile Terminated)	4

2.2 SMS-SUBMIT (Mobile Originated)	5

3. Parameter description	6

3.1 Protocol Data Unit Type (PDU Type)	6

3.2 Message Reference MR	7

3.3 Originator Adress OA Destination Adress DA	7

3.4 Protocol Identifier PID	8

3.5 Data Coding Scheme DCS	8

3.6 Service Center Time Stamp SCTS	9

3.7 Validity Period VP	9

3.8 User Data Length UDL and User Data UD	10

4. PDU Examples	11

5. Appendix	12

# 1. Introduction

To use the SMS you have to declare the number of the SMSC (Short Message Service Center) in the MS (Mobile Station), provided that the MS support SMS-MO (Short Message Service-Mobile Orginated).

The S3 com, the S4 and the M1 for example are providing SMS-MO.

| card | SMSC-number (Germany) |
| --- | --- |
| D1 | 491710760000 |
| D2 | 491722270000 |

At the M1 you enter the SMSC-number with the AT+Celular command:

AT+csca = ”<SMSC-number>”

If the receiver of the SMS possesses a D2 card, the AT command has to be entered in the following way:

AT+csca = "+491722270000"

With the command

AT+csca?

you can question the actual adjusted SMSC-number

Ask your network operator for the right SMSC-number !!

! notice: The SMSC-number is not a part of the Protocol Data Unit (PDU) !

# 2. Overview:

MS: Mobile Station

SME: Short Message Entity

SMSC: Short Message Service Center

MMI: Man Machine Interface

PDUs: Protocol Data Units

SM-AL: Short Message Aplication Layer

SM-TL: Short Message Transport Layer

SM-RL: Short Message Relay Layer

SM-LL: Short Message Link Layer

The MMI is based on the command set of AT+Cellular, and could be realized by means of a terminal (for example Triodata, Telix, WIN-Terminal) or the display of a handy.

The SM-TL provides a service to the Short Message Application Layer. This service enables the SM-AL to transfer short messages to its peer entity, receive short messages from its peer entity and receive reports about earlier requests for short messages to be transferred.

The SM-TL communicates with its peer entity with six several PDUs (Protocol Data Units):

	SMS-DELIVER, conveying a short message from the SMSC to the MS

	SMS-DELIVER-REPORT, conveying a failure cause (if necessary)

	SMS-SUBMIT, conveying a short message from the MS to the SMSC

	SMS-SUBMIT-REPORT, conveying a failure cause (if necessary)

	SMS-STATUS-REPORT, conveying a status report from the SMSC to the MS

	SMS-COMMAND, conveying a command from the MS to the SMSC

At this time the M1 is supporting the SMS-DELIVER and SMS-SUBMIT PDUs described in the following sections.

## 2.1 SMS-DELIVER (Mobile Terminated)

MTI	bit 1 = 0

bit 0 = 0

## 2.2 SMS-SUBMIT (Mobile Originated)

MTI	bit 1 = 0

bit 0 = 1

! notice: Any unused bits will be set to zero by the sending entity and will be ignored by 		the receiving entity !

| PDU Type | Protocol Data Unit Type |  |
| --- | --- | --- |
| MR | Message Reference | sucessive number (0..255) of all SMS-SUBMIT Frames set by the M1 |
| OA | Originator Adress | Adress of the originating SME |
| DA | Destination Adress | Adress of the destination SME |
| PID | Protocol Identifier | Parameter showing the SMSC how to process the SM (as FAX, Voice etc) |
| DCS | Data Coding Scheme | Parameter identifying the coding scheme within the User Data (UD) |
| SCTS | Service Center Time Stamp | Parameter identifying time when the SMSC received the message |
| VP | Validity Period | Parameter identifying the time from where the message is no longer valid in the SMSC |
| UDL | User Data Length | Parameter indicating the length of the UD-field |
| UD | User Data | Data of the SM |
| RP | Reply Path | Parameter indicating that Replay Path exists |
| UDHI | User Data Header Indicator | Parameter indicating that the UD field contains a header |
| SRI | Status Report Indication | Parameter indicating if the SME has requested a status report |
| SRR | Status Report Request | Parameter indicating if the MS has requested a status report |
| VPF | Validity Period Format | Parameter indicating whether or not the VP field is present |
| MMS | More Messages to Send | Parameter indicating whether or not there are more messages to send |
| RD | Reject Duplicate |  |
| MTI | Message Type Indicator | Parameter describing the message type 00 means SMS-DELIVER 01 means SMS-SUBMIT |

# 3. Parameter description

## 3.1 Protocol Data Unit Type (PDU Type)

SMS-SUBMIT:					SMS-DELIVER:

! notice: you have to write the PDU-type in Hex-Format, a possible example is ”11H” !

RP:	0	Replay Path parameter is not set in this PDU

1	Replay Path parameter is set in this PDU

UDHI:	0	The UD field contains only the short message

1	The beginning of the UD field contains a header in addition of the short 				message

SRI: (is only set by the SMSC)

0	A status report will not be returned to the SME

1	A status report will be returned to the SME

SRR:	0	A status report is not requested

1	A status report is requested

VPF:	bit4	bit3

0	0	VP field is not present

0	1	Reserved

1	0	VP field present an integer represented (relative)

1	1	VP field present an semi-octet represented (absolute)

any reserved values may be rejected by the SMSC

MMS: (is only set by the SMSC)

0	More messages are waiting for the MS in the SMSC

1	No more messages are waiting for the MS in the SMSC

RD:	0	Instruct the SMSC to accept an SMS-SUBMIT for an short message still 			held in the SMSC which has the same MR and DA as a previosly 				submitted short message from the same OA.

1	Instruct the SMSC to reject an SMS-SUBMIT for an short message still held 			in the SMSC which has the same MR and DA as a previosly submitted short 			message from the same OA.

MTI:	bit1	bit0	Message type

0	0	SMS-DELIVER (SMSC ==> MS)

0	0	SMS-DELIVER REPORT (MS ==> SMSC)

0	1	SMS-SUBMIT (MS ==> SMSC)

0	1	SMS-SUBMIT REPORT (SMSC ==> MS)

1	0	SMS-STATUS REPORT (SMSC ==> MS)

1	0	SMS-COMMAND (MS ==> SMSC)

1	1	Reserved

(the fat-marked lines represent the features supported by the M1)

## 3.2 Message Reference MR

The MR field gives an integer (0..255) representation of a reference number of the SMS-SUBMIT submitted to the SMSC by the MS.

! notice: at the M1 the MR is generated automatically, -anyway you have to generate it-

a possible entry is for exaple ”00H” !

## 3.3 Originator Adress OA Destination Adress DA

OA and DA have the same format explained in the following lines:

len:

The octet ”len” contains the number of BCD digits

type of number:

81H: the following number is national

91H: the following number international

(for further information see GSM 04.08 chapter 10.5.4.6)

BCD-digits:

The BCD-digit Field contains the BCD-number of the Destination e.g. of the Originator

If the called party BCD number contains an odd number of digits, the last digit shall be filled with an end mark coded as ”FH”

Example:

if you have the national number 1234567 you have to type:

0781214365F7

## 3.4 Protocol Identifier PID

The PID is the information element by which the Transport Layer either refers to the higher layer protocol being used, or indicates interworking with a certain type of telematic device.

here are some examples of PID codings:

00H: The PDU has to be treat as a short message

01H: The PDU has to be treat as a telex

02H: The PDU has to be treat as group3 telefax

03H: The PDU has to be treat as group4 telefax

(for further information see GSM 03.40 chapter 9.2.3.9)

! notice: it is not guaranteed that the SMSC supports every PID codings!

## 3.5 Data Coding Scheme DCS

The DCS field indicates the data coding scheme of the UD (User Data) field, and may indicate a message class. the octet is used according to a coding group which is indicated in bits 7..4. The octet is then coded as follows:

| Coding group: bits 7..4 | bits 3..0 |
| --- | --- |
| 0000 | Alphabet indication Unspecified message handling at the MS 0000 Default alphabet (7 bit data coding in the User Data) 0001-1111 reserved |
| 0001-1110 | Reserved coding groups |
| 1111 | Data Coding/message class bit 3 is reserved, set 0 bit 2 (message coding) 0 Default alphabet (7 bit data coding in the User Data) 1 8-bit data coding in the User Data bit 1 bit 0 (message class) 0 0 Class0 immediate display 0 1 Class1 ME (Mobile Equipment)- specific 1 0 Class2 SIM specific message 1 1 Class3 TE (Terminate Equipment)- specific |

Default alphabet indicates that the UD (User Data) is coded from the 7-bit alphabet given in the appendix. When this alphabet is used, eight characters of the message are packed in seven octets, and the message can consist of up to 160 characters (instead of 140 characters in 8-bit data coding)

In 8-bit data coding, you can relate to the INTEL ASCII-HEX table.

In Class 0 (immediate display) the short message is written directly in the display, as the M1 has no display the Class 0 message can be realised only in a roundabout way.

In Class 1 to Class 3 the short message is stored in the several equipments ME, SIM-card and TE.

In time the Class 2 is supported, if you choose Class 1 or Class 3 the short message is treated the same way as a Class 2 message.

! note: It is recommended to use the Class2 message, or the coding group ”0000 bin” !

## 3.6 Service Center Time Stamp SCTS

The SCTS is the information element by which the SMSC informs the recipient MS about the time of arrival of the short message at the Transport Layer entity of the SMSC. The time value is included in every SMS-DELIVER being delivered to the SMSC, and represents the local time in the following way:

The Time Zone indicates the difference, expressed in quarters of an hour, between the local time and GMT (Greenwich Main Time).

## 3.7 Validity Period VP

The Validity-Period is the information element which gives an MS submitting an SMS-SUBMIT to the SMSC the possibility to include a specific time period value in the short message. The Validity Period parameter value indicates the time period for which the short message is valid, i.e. for how long the SMSC shall guarantee its existence in the SMSC memory before delivery to the recipient has been carried out.

The VP field is given in either integer or semi-octet representation. In the first case, the VP comprises 1 octet, giving the length of the validity period, counted from when the SMS-SUBMIT is received by the SMSC. In the second case, the VP comprises 7 octets, giving the absolute time of the vality period termination.

In the first case, the representation of time is as follows:

| VP Value | Validity period value |
| --- | --- |
| 0-143 | (VP + 1) x 5 minutes (i.e 5 minutes intervals up to 12 hours) |
| 144-167 | 12 hours + ((VP-143) x 30 minutes) |
| 168-196 | (VP-166) x 1 day |
| 197-255 | (VP - 192) x 1 week |

in the second case, the representation of time is identical to the representation or the SCTS (Service Center Time Stamp)

The case of representation is set in the VPF (Validity Period Format) in the PDU-type.

## 3.8 User Data Length UDL and User Data UD

The UDL field gives an integer representation of the number of characters within the User Data field to follow.

# 4. PDU Examples

here are two examples how to send a short message with AT+Cellular:

first enter PIN-number and the Service Center Adress:

at+cpin="XXXX"	enter the PIN-number

OK

at+csca="+491722270000"	enter the Service-Center-Adress (here D2)

OK

1st example:

at+cmgs=140	enter “send message”, 140 is the maximum length (in 	byte) of the PDU

> 11000781214365F70000AA05E8329BFD06	type the PDU (SMS-SUBMIT) and finish with “ctrl Z”	the thin-typed characters are the Destination Adress

e.g. the own tel.-number

+CMGS: 0

OK

at+cpms?	are messages stored on the SIM-Card?

+CPMS: "SM" , 1 , 7 , "SM" , 1 , 7	on this SIM-Card is 1 message stored

OK	you can store at most 7 messages

at+cmgr=1	read stored message in location 1

+CMGR: 0 , 24

040C9194718215219200006930824161840005E8329BFD06	This is a PDU (SMS-DELIVER) sent by the

OK			Service Center

2nd example:

at+cmgw=140	write message in the memory of the SIM-card

> 11000781214365F700F6AA0568656C6C6F	type the PDU (SMS-SUBMIT) and finish with “ctrl Z”	the thin-typed characters are the Destination Adress

e.g. the own tel.-number

+CMGW: 2

OK

at+cmgr=2	read stored message in location 2

+CMGR: 2 , 17

11000781214365F700F6AA0568656C6C6F	this is the PDU stored in location 2

OK

at+cmss=2	send the message stored in location 2

+CMSS: 3

OK

at+cpms?	are messages stored on the SIM-Card?

+CPMS: "SM" , 3 , 7 , "SM" , 3 , 7 	on this SIM-Card are 3 message stored

OK	you can store at most 7 messages

at+cmgr=3	read stored message in location 3

+CMGR: 0 , 24

040C9194718215219200F6693082519472000568656C6C6F 	This is a PDU (SMS-DELIVER) sent by the

OK			Service Center

OK

notice: don´t dial the area code sending messages between the same networks

# 5. Appendix

Default alphabet:

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 2 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ | D | SP | 0 |  | P | ¨ | p |
| 0 | 0 | 0 | 1 | 1 |  |  | ! | 1 | A | Q | a | q |
| 0 | 0 | 1 | 0 | 2 | $ | F | ² | 2 | B | R | b | r |
| 0 | 0 | 1 | 1 | 3 |  | G | # | 3 | C | S | c | s |
| 0 | 1 | 0 | 0 | 4 |  | L |  | 4 | D | T | d | t |
| 0 | 1 | 0 | 1 | 5 |  | W | % | 5 | E | U | e | u |
| 0 | 1 | 1 | 0 | 6 |  | P | & | 6 | F | V | f | v |
| 0 | 1 | 1 | 1 | 7 |  | Y | ‘ | 7 | G | W | g | w |
| 1 | 0 | 0 | 0 | 8 |  | S | ( | 8 | H | X | h | x |
| 1 | 0 | 0 | 1 | 9 |  | Q | ) | 9 | I | Y | i | y |
| 1 | 0 | 1 | 0 | 10 | LF | X | * | : | J | Z | j | z |
| 1 | 0 | 1 | 1 | 11 |  |  | + | ; | K | Ä | k | ä |
| 1 | 1 | 0 | 0 | 12 |  |  | , | < | L | Ö | l | ö |
| 1 | 1 | 0 | 1 | 13 | CR |  | - | = | M |  | m |  |
| 1 | 1 | 1 | 0 | 14 |  | ß | . | > | N | Ü | n | ü |
| 1 | 1 | 1 | 1 | 15 |  |  | / | ? | O |  | o |  |

abbreviations:

MS	Mobile Station

SME	Short Message Entity

SMSC	Short Message Service Center

MMI	Man Machine Interface

PDUs	Protocol Data Units

SM-AL	Short Message Aplication 	Layer

SM-TL	Short Message Transport 	Layer

SM-RL	Short Message Relay Layer

SM-LL	Short Message Link Layer

PDU Type	Protocol Data Unit Type

MR	Message Reference

OA	Originator Adress

DA	Destination Adress

PID	Protocol Identifier

DCS	Data Coding Scheme

SCTS	Service Center Time Stamp

VP	Validity Period

UDL	User Data Length

UD	User Data

RP	Reply Path

UDHI	User Data Header Indicator

SRI	Status Report Indication

SRR	Status Report Request

VPF	Validity Period Format

MMS	More Messages to Send

RD	Reject Duplicate

MTI	Message Type Indicator

ME	Mobile Equipment

TE	Terminal Equipment

SIM	Subscriber Identification 	Modul

sometimes you can see the abbreviation SC (Service Center) that means the same as SMSC